COSHH Assessment Hollins & Reed Contracting Ltd

| Project or site House refurbishment, House 7 | Assessment ref CA-006 | Date assessed 5 October 2026 |
| --- | --- | --- |

| Substance |  |  |  |
| --- | --- | --- | --- |
| Product or substance | Aerosol penetrating lubricant | Supplier | your supplier |
| SDS date | your sheet’s date | Form (liquid, dust…) | Aerosol: light oil in propellant |

| The work |  |  |  |
| --- | --- | --- | --- |
| Task | Freeing bolts and hinges | Where | Around the site |
| How often, how long | A few times a week, seconds each | Amount, used with | One 400 ml can a month |

| Hazards (H codes) | Harmful or irritant | Corrosive | Toxic | Serious health hazard | Cancer or asthma | Flammable |
| --- | --- | --- | --- | --- | --- | --- |
| H222, H229, H220, H304 | ☐ | ☐ | ☐ | ☒ | ☐ | ☒ |

| How it gets in | Breathing in | Skin | Eyes | Swallowing | Puncture |
| --- | --- | --- | --- | --- | --- |
| Mist, skin | ☒ | ☒ | ☒ | ☐ | ☐ |

| Who could be harmed | People doing it | Others nearby | Cleaners, fitters | Visitors, public | Higher risk |
| --- | --- | --- | --- | --- | --- |
| Sprayer, others close by | ☒ | ☒ | ☐ | ☐ | ☐ |

Health effects

Extremely flammable. The can may burst if heated. Gas can collect and ignite in a tight space.

Propellant limits: butane 600 ppm over 8 hours, 750 over 15 minutes; LPG 1000 and 1250. H304 is the base oil.

| Controls | In place | Still needed |
| --- | --- | --- |
| Avoid or swap it | Grease or brush-on oil where they will do. | Trial brush-on oil. |
| Change the method | Short bursts. Never on hot or live parts. |  |
| Extract at source | Open air. Not in a cupboard or trench. |  |
| Fewer, for less time | Named people, one can each. |  |
| Washing, welfare | Soap and water at the cabin. |  |

| PPE: type or standard | Gloves | Eye protection | RPE | Overalls | Boots |
| --- | --- | --- | --- | --- | --- |
| Safety glasses, gloves to your sheet. | ☒ | ☒ | ☐ | ☐ | ☐ |

| If it goes wrong |  |
| --- | --- |
| Spill or leak | Move leaking cans outdoors. Keep off sparks and heat. |
| Fire | Clear the area, call 999: cans may burst. Extinguisher only if trained and it is safe. |
| First aid | Skin: wash. Eyes: flush at least 10 minutes, then hospital. Swallowed: your sheet, call 999. |

| Further action | Who | By when | Checked |
| --- | --- | --- | --- |
| Add aerosols to the DSEAR assessment, stored away from heat | DH | Before start |  |

| Assessed by Dan Hollins | Signed | Date 5 October 2026 | Next review 5 October 2027 |
| --- | --- | --- | --- |