COSHH Assessment Hollins & Reed Contracting Ltd

| Project or site House refurbishment, House 7 | Assessment ref CA-001 | Date assessed 5 October 2026 |
| --- | --- | --- |

| Substance |  |  |  |
| --- | --- | --- | --- |
| Product or substance | Cement, mortar, concrete, wet | Supplier | your supplier |
| SDS date | your sheet’s date | Form (liquid, dust…) | Powder, then wet paste |

| The work |  |  |  |
| --- | --- | --- | --- |
| Task | Mortar, blocks, small pours | Where | Rear extension, outdoors |
| How often | Most working days | How long each time | Up to 6 hours a day |
| Amount used | About 10 bags of 25 kg a week | Used with | Sand, water |

| Storage and waste |  |
| --- | --- |
| Storage | Dry, off the ground, oldest first: the chromate reducer only works for a limited time. |
| Handling | Bags of 25 kg: two people or a barrow. Open bags outdoors. |
| Waste | Set waste as builders’ rubble. Empty bags in a sealed sack. |

| Hazards (H codes) | Harmful or irritant | Corrosive | Toxic | Serious health hazard | Cancer or asthma | Flammable |
| --- | --- | --- | --- | --- | --- | --- |
| No GB list entry. Your sheet gives the codes. | ☒ | ☒ | ☐ | ☒ | ☐ | ☐ |

| Exposure limit |  |  |  |
| --- | --- | --- | --- |
| WEL, 8-hour | Inhalable 10, respirable 4 mg/m³ | WEL, 15-minute | None listed |
| Monitoring result | Not measured | Measured on | Not measured |

| How it gets in | Breathing in | Skin | Eyes | Swallowing | Puncture |
| --- | --- | --- | --- | --- | --- |
| Skin, then dust from bags | ☒ | ☒ | ☒ | ☐ | ☐ |

| Who could be harmed | People doing it | Others nearby | Cleaners, fitters | Visitors, public | Higher risk |
| --- | --- | --- | --- | --- | --- |
| Bricklayer, labourer. Eczema or allergy. | ☒ | ☐ | ☐ | ☐ | ☒ |

Health effects

Wet cement is highly alkaline: trapped on the skin it can burn quickly, and it can burn the eyes.

Irritant and allergic dermatitis. Once sensitised, any exposure may trigger it. Dust irritates nose and throat.

| Risk of ill health | Negligible | Remote | Possible | Probable | Minor | Significant | Serious |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Before controls | ☐ | ☐ | ☐ | ☒ | ☐ | ☒ | ☐ |
| With controls | ☐ | ☒ | ☐ | ☐ | ☐ | ☒ | ☐ |

COSHH Assessment Hollins & Reed Contracting Ltd

| Project or site House refurbishment, House 7 | Assessment ref CA-001 | Date assessed 5 October 2026 |
| --- | --- | --- |

| Controls | In place | Still needed |
| --- | --- | --- |
| Avoid or swap it | Ready-mixed for pours. Site mixer only for mortar. | Ask for pre-mixed mortar next. |
| Change the method | Long-handled tools. Knee boards when screeding. |  |
| Extract at source | Not used: open air. Open bags outdoors, slowly. |  |
| Fewer, for less time | Two people only on mixing. |  |
| Washing, welfare | Warm water, soap, towels. Wash at breaks and after work. | Skin cream at the cabin. |

| PPE: type or standard | Gloves | Eye protection | RPE | Overalls | Boots |
| --- | --- | --- | --- | --- | --- |
| EN 374 nitrile or PVC gloves. Trousers over boots. | ☒ | ☒ | ☐ | ☒ | ☒ |

| If it goes wrong |  |
| --- | --- |
| Spill or leak | Scrape up, keep out of drains, let it set, bag it. |
| Fire | Does not burn. |
| Breathed in or swallowed | Move to fresh air. Rinse mouth with water. |
| On skin or in eyes | Skin: wash off, flood for 20 minutes, treat as a burn. Eyes: flush at least 10 minutes, then hospital. |

| Also needed | Yes | No | Detail | Next due |
| --- | --- | --- | --- | --- |
| Exposure monitoring | ☐ | ☒ | Dust is brief and outdoors. |  |
| Health surveillance | ☒ | ☐ | Skin checks by a responsible person. | Weekly |
| LEV examination | ☐ | ☒ | None used. |  |
| RPE face-fit test | ☐ | ☒ | None used. |  |
| Training, supervision | ☒ | ☐ | Dermatitis signs, washing, gloves. | Before start |

| Further action | Who | By when | Checked |
| --- | --- | --- | --- |
| Train a responsible person for skin checks | DH | Before start |  |
| Baseline skin check on both workers | DH | Before start |  |

| Date | By | Review: reason and changes | Next review |
| --- | --- | --- | --- |
| 5 October 2026 | DH | First issue | 5 October 2027 |

| Assessed by Dan Hollins | Signed | Date 5 October 2026 | Shared with workers on 6 October 2026 |
| --- | --- | --- | --- |