COSHH Assessment Hollins & Reed Contracting Ltd

| Project or site House refurbishment, House 7 | Assessment ref CA-012 | Date assessed 5 October 2026 |
| --- | --- | --- |

| Substance |  |  |  |
| --- | --- | --- | --- |
| Product or substance | Solvent contact adhesive | Supplier | your supplier |
| SDS date | your sheet’s date | Form (liquid, dust…) | Liquid that gives off vapour |

| The work |  |  |  |
| --- | --- | --- | --- |
| Task | Sticking flooring, laminate | Where | Kitchen and utility room |
| How often | One day on this job | How long each time | Up to 2 hours |
| Amount used | About 2.5 litres | Used with | Notched spreader, brush |

| Storage and waste |  |
| --- | --- |
| Storage | Sealed tins in the outdoor lock-up, away from heat and sun. Only the tin in use indoors. |
| Handling | Lids back on straight away. Spread one area at a time. No smoking, flames or pilot lights in the room. |
| Waste | Hazardous waste, not builders’ rubble: wet tins, spreaders and rags, sealed, by a registered carrier. Never down a drain. |

| Hazards (H codes) | Harmful or irritant | Corrosive | Toxic | Serious health hazard | Cancer or asthma | Flammable |
| --- | --- | --- | --- | --- | --- | --- |
| H225 or H226, H315, H319, H336. Some add H317. | ☒ | ☐ | ☐ | ☐ | ☐ | ☒ |

| Exposure limit |  |  |  |
| --- | --- | --- | --- |
| WEL, 8-hour | Acetone 500, butanone 200, ethyl acetate 200 ppm | WEL, 15-minute | Acetone 1500, butanone 300, ethyl acetate 400 ppm |
| Monitoring result | Not measured | Measured on | Not measured |

| How it gets in | Breathing in | Skin | Eyes | Swallowing | Puncture |
| --- | --- | --- | --- | --- | --- |
| Vapour first, then skin | ☒ | ☒ | ☒ | ☒ | ☐ |

| Who could be harmed | People doing it | Others nearby | Cleaners, fitters | Visitors, public | Higher risk |
| --- | --- | --- | --- | --- | --- |
| Fitters, other trades. Eczema or allergy. | ☒ | ☒ | ☐ | ☐ | ☒ |

Health effects

Highly flammable: vapour can reach a flame or pilot light and flash back. It can build up in a small room.

Vapour causes drowsiness and dizziness. Irritates eyes and skin. Your sheet may warn of a skin allergy.

| Risk of ill health | Negligible | Remote | Possible | Probable | Minor | Significant | Serious |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Before controls | ☐ | ☐ | ☒ | ☐ | ☐ | ☐ | ☒ |
| With controls | ☐ | ☒ | ☐ | ☐ | ☐ | ☐ | ☒ |

COSHH Assessment Hollins & Reed Contracting Ltd

| Project or site House refurbishment, House 7 | Assessment ref CA-012 | Date assessed 5 October 2026 |
| --- | --- | --- |

| Controls | In place | Still needed |
| --- | --- | --- |
| Avoid or swap it | Water-based on sheet flooring where it will hold. | Try water-based next job. |
| Change the method | Spreader, no spraying. Pilot lights and heaters off first. |  |
| Extract at source | Not used. Windows and the door open throughout. | Room that will not air: stop, ask. |
| Fewer, for less time | One fitter in the room. Out while it dries. |  |
| Washing, welfare | Soap and water at the cabin. No solvent on hands. | Skin cream at the cabin. |

| PPE: type or standard | Gloves | Eye protection | RPE | Overalls | Boots |
| --- | --- | --- | --- | --- | --- |
| Gloves per sheet, EN 374. Glasses. | ☒ | ☒ | ☐ | ☒ | ☒ |

| If it goes wrong |  |
| --- | --- |
| Spill or leak | Flames out, windows open. Soak up with sand, bag it as hazardous waste. Keep it out of drains. |
| Fire | Raise the alarm, get out, ring 999. Only if safe, a dry powder or CO2 extinguisher. No water jet. |
| Breathed in or swallowed | Fresh air and rest. Swallowed: do not make them sick. Rinse the mouth, get medical help with the sheet. |
| On skin or in eyes | Skin: wash with soap and water. Eyes: rinse at least 15 minutes, get medical help. |

| Also needed | Yes | No | Detail | Next due |
| --- | --- | --- | --- | --- |
| Exposure monitoring | ☐ | ☒ | Up to 2 hours, room aired. Dizzy: out. |  |
| Health surveillance | ☒ | ☐ | Skin checks by a responsible person. | Weekly |
| LEV examination | ☐ | ☒ | None used. |  |
| RPE face-fit test | ☐ | ☒ | None used. |  |
| Training, supervision | ☒ | ☐ | Fire, vapour, skin, when to stop. | Before start |

| Further action | Who | By when | Checked |
| --- | --- | --- | --- |
| Extinguisher at the door and DSEAR assessment done | DH | Before start |  |
| Lidded metal rag bin, emptied outdoors daily | DH | Before start |  |
| Book a registered carrier for the waste | DH | Before start |  |

| Date | By | Review: reason and changes | Next review |
| --- | --- | --- | --- |
| 5 October 2026 | DH | First issue | 5 October 2027 |

| Assessed by Dan Hollins | Signed | Date 5 October 2026 | Shared with workers on 6 October 2026 |
| --- | --- | --- | --- |