COSHH Assessment Hollins & Reed Contracting Ltd

| Project or site House refurbishment, House 7 | Assessment ref CA-003 | Date assessed 5 October 2026 |
| --- | --- | --- |

| Substance |  |  |  |
| --- | --- | --- | --- |
| Product or substance | Diesel | Supplier | your supplier |
| SDS date | your sheet’s date | Form (liquid, dust…) | Liquid |

| The work |  |  |  |
| --- | --- | --- | --- |
| Task | Filling plant and heater | Where | Bunded bowser by the compound |
| How often, how long | Daily, about 10 minutes | Amount, used with | Up to 40 litres, nothing mixed in |

| Hazards (H codes) | Harmful or irritant | Corrosive | Toxic | Serious health hazard | Cancer or asthma | Flammable |
| --- | --- | --- | --- | --- | --- | --- |
| H351. See your sheet. | ☐ | ☐ | ☐ | ☒ | ☒ | ☒ |

| How it gets in | Breathing in | Skin | Eyes | Swallowing | Puncture |
| --- | --- | --- | --- | --- | --- |
| Skin contact, mist, exhaust | ☒ | ☒ | ☒ | ☐ | ☐ |

| Who could be harmed | People doing it | Others nearby | Cleaners, fitters | Visitors, public | Higher risk |
| --- | --- | --- | --- | --- | --- |
| Filler, others near plant | ☒ | ☒ | ☐ | ☐ | ☐ |

Health effects

Suspected of causing cancer. Repeated skin contact can cause dermatitis.

No EH40 limit for diesel. Exhaust is a risk of its own, worst in enclosed spaces and with blue or black smoke.

| Controls | In place | Still needed |
| --- | --- | --- |
| Avoid or swap it | Mains power where it is available. | Ask for mains on the next phase. |
| Change the method | Pump with auto-shut nozzle. Engine off. |  |
| Extract at source | Run plant outdoors. Exhaust away from doors. |  |
| Fewer, for less time | One person fills. Plant serviced, no smoke. |  |
| Washing, welfare | Soap and water. No oily rags in pockets. |  |

| PPE: type or standard | Gloves | Eye protection | RPE | Overalls | Boots |
| --- | --- | --- | --- | --- | --- |
| Gloves per sheet, safety glasses. | ☒ | ☒ | ☐ | ☐ | ☐ |

| If it goes wrong |  |
| --- | --- |
| Spill or leak | Spill kit, keep out of drains. Storing 201 litres or more in England needs a 110% bund. |
| Fire | Stop plant, clear the area, call 999. Extinguisher only if trained and it is safe. |
| First aid | Skin: wash. Eyes: flush at least 10 minutes, hospital. Swallowed: 999, your sheet. |

| Further action | Who | By when | Checked |
| --- | --- | --- | --- |
| Check the bowser’s bund and spill kit | DH | Before start |  |

| Assessed by Dan Hollins | Signed | Date 5 October 2026 | Next review 5 October 2027 |
| --- | --- | --- | --- |