COSHH Assessment Hollins & Reed Contracting Ltd

| Project or site House refurbishment, House 7 | Assessment ref CA-014 | Date assessed 5 October 2026 |
| --- | --- | --- |

| Substance |  |  |  |
| --- | --- | --- | --- |
| Product or substance | Gun-grade PU foam, MDI | Supplier | your supplier |
| SDS date | your sheet’s date | Form (liquid, dust…) | Pressurised foam |

| The work |  |  |  |
| --- | --- | --- | --- |
| Task | Sealing new window frames | Where | Indoors, window open beside it |
| How often, how long | Few times a year, under 1 hour | Amount, used with | A few cans, nothing mixed in |

| Hazards (H codes) | Harmful or irritant | Corrosive | Toxic | Serious health hazard | Cancer or asthma | Flammable |
| --- | --- | --- | --- | --- | --- | --- |
| Main: H222, H334, H351 | ☒ | ☐ | ☐ | ☒ | ☒ | ☒ |

| How it gets in | Breathing in | Skin | Eyes | Swallowing | Puncture |
| --- | --- | --- | --- | --- | --- |
| Vapour, skin, eyes | ☒ | ☒ | ☒ | ☐ | ☐ |

| Who could be harmed | People doing it | Others nearby | Cleaners, fitters | Visitors, public | Higher risk |
| --- | --- | --- | --- | --- | --- |
| Fitter, anyone nearby | ☒ | ☒ | ☐ | ☐ | ☒ |

Health effects

May cause asthma and skin allergy: once allergic, a trace can set it off. Suspected of causing cancer. Extremely flammable.

EH40 isocyanates 0.02 mg/m³, 0.07 over 15 minutes. Skin checks each job. Yearly breathing questionnaire, health record.

| Controls | In place | Still needed |
| --- | --- | --- |
| Avoid or swap it | Foam tape where it will do. | Try tape on the next job. |
| Change the method | Gun, not a straw. A bead only. Cool, no flames. |  |
| Extract at source | Not used. Window beside it and door open. | Room that will not air: stop, ask. |
| Fewer, for less time | One fitter. Nobody with asthma or eczema near. |  |
| Washing, welfare | Soap and water at breaks. No solvent on skin. | Skin cream at the cabin. |

| PPE: type or standard | Gloves | Eye protection | RPE | Overalls | Boots |
| --- | --- | --- | --- | --- | --- |
| Nitrile gloves, goggles, overalls. | ☒ | ☒ | ☐ | ☒ | ☐ |

| If it goes wrong |  |
| --- | --- |
| Spill or leak | Wet foam: wipe up with a cloth. Cured: cut away. Bag as hazardous waste. |
| Fire | Get out, ring 999. Cans can burst. Extinguisher only if trained and it is safe. |
| First aid | Skin, wet or cured: soap, water, brush, no solvent. Eyes: rinse 15 minutes. Wheeze: doctor. |

| Further action | Who | By when | Checked |
| --- | --- | --- | --- |
| Isocyanate training recorded, breathing checks set up | DH | Before start |  |

| Assessed by Dan Hollins | Signed | Date 5 October 2026 | Next review 5 October 2027 |
| --- | --- | --- | --- |