COSHH Assessment Hollins & Reed Contracting Ltd

| Project or site House refurbishment, House 7 | Assessment ref CA-010 | Date assessed 5 October 2026 |
| --- | --- | --- |

| Substance |  |  |  |
| --- | --- | --- | --- |
| Product or substance | Hydraulic oil, mineral | Supplier | your supplier |
| SDS date | your sheet’s date | Form (liquid, dust…) | Oily liquid |

| The work |  |  |  |
| --- | --- | --- | --- |
| Task | Topping up, changing hoses | Where | Outdoors, machine on level ground |
| How often, how long | Weekly, about 10 minutes | Amount, used with | Up to 5 litres, nothing mixed in |

| Hazards (H codes) | Harmful or irritant | Corrosive | Toxic | Serious health hazard | Cancer or asthma | Flammable |
| --- | --- | --- | --- | --- | --- | --- |
| None on most sheets. | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ |

| How it gets in | Breathing in | Skin | Eyes | Swallowing | Puncture |
| --- | --- | --- | --- | --- | --- |
| Skin, mist, jet through skin | ☒ | ☒ | ☒ | ☐ | ☒ |

| Who could be harmed | People doing it | Others nearby | Cleaners, fitters | Visitors, public | Higher risk |
| --- | --- | --- | --- | --- | --- |
| Fitter, anyone nearby | ☒ | ☒ | ☐ | ☐ | ☐ |

Health effects

A jet from a pinhole leak can go through skin and look like nothing. Without quick treatment it can cost a limb, or a life.

Repeated contact with oil can cause dermatitis. No EH40 limit for oil mist.

| Controls | In place | Still needed |
| --- | --- | --- |
| Avoid or swap it | Fitter changes hoses. Operator only tops up. | Add hoses to the daily check. |
| Change the method | Engine off, boom down, pressure released first. |  |
| Extract at source | Not needed: outdoors, engine off. |  |
| Fewer, for less time | Never feel for a leak by hand. Use card. | Card kept on the machine. |
| Washing, welfare | Soap and water at the cabin. Change oily clothes. | Skin cream at the cabin. |

| PPE: type or standard | Gloves | Eye protection | RPE | Overalls | Boots |
| --- | --- | --- | --- | --- | --- |
| Nitrile gloves, safety glasses, overalls. | ☒ | ☒ | ☐ | ☒ | ☒ |

| If it goes wrong |  |
| --- | --- |
| Spill or leak | Engine off. Spill kit, keep out of drains, bag as hazardous waste. |
| Fire | Raise the alarm, ring 999. Foam, CO2 or powder extinguisher only if safe. |
| First aid | Jet through skin: hospital now, even if it looks small. Say hydraulic injection injury. |

| Further action | Who | By when | Checked |
| --- | --- | --- | --- |
| Spill kit, card and skin cream on site | DH | Before start |  |

| Assessed by Dan Hollins | Signed | Date 5 October 2026 | Next review 5 October 2027 |
| --- | --- | --- | --- |