COSHH Assessment Hollins & Reed Contracting Ltd

| Project or site House refurbishment, House 7 | Assessment ref CA-002 | Date assessed 5 October 2026 |
| --- | --- | --- |

| Substance |  |  |  |
| --- | --- | --- | --- |
| Product or substance | Petrol (unleaded) | Supplier | your supplier |
| SDS date | your sheet’s date | Form (liquid, dust…) | Liquid and vapour |

| The work |  |  |  |
| --- | --- | --- | --- |
| Task | Refuelling saw and generator | Where | Outdoors, marked refuelling spot |
| How often, how long | Daily, under 5 minutes each | Amount, used with | Up to 5 litres, nothing mixed in |

| Hazards (H codes) | Harmful or irritant | Corrosive | Toxic | Serious health hazard | Cancer or asthma | Flammable |
| --- | --- | --- | --- | --- | --- | --- |
| H304, H340, H350. | ☐ | ☐ | ☐ | ☒ | ☒ | ☒ |

| How it gets in | Breathing in | Skin | Eyes | Swallowing | Puncture |
| --- | --- | --- | --- | --- | --- |
| Vapour, splashes, siphoning | ☒ | ☒ | ☒ | ☒ | ☐ |

| Who could be harmed | People doing it | Others nearby | Cleaners, fitters | Visitors, public | Higher risk |
| --- | --- | --- | --- | --- | --- |
| Refueller, anyone nearby | ☒ | ☒ | ☐ | ☐ | ☐ |

Health effects

May cause cancer and genetic defects. May be fatal if swallowed and enters the airways. Flammable: code on your sheet.

Treat as a carcinogen: petrol contains benzene. No EH40 limit for petrol; benzene 1 ppm over 8 hours.

| Controls | In place | Still needed |
| --- | --- | --- |
| Avoid or swap it | Battery or mains tools where the job allows. | Price a battery cut-off saw. |
| Change the method | Engine off, cool. Spout. No smoking or flames. | Extinguisher at the spot. |
| Extract at source | Open air only. Not in a van, cabin or trench. |  |
| Fewer, for less time | One trained person. UN-approved cans, shut. | Cans in a ventilated lock-up. |
| Washing, welfare | Wash splashes off. Change splashed clothes. |  |

| PPE: type or standard | Gloves | Eye protection | RPE | Overalls | Boots |
| --- | --- | --- | --- | --- | --- |
| Gloves per sheet, safety glasses. | ☒ | ☒ | ☐ | ☐ | ☐ |

| If it goes wrong |  |
| --- | --- |
| Spill or leak | Stop engines and flames. Spill kit, keep out of drains, bag as hazardous waste. |
| Fire | Clear the area, call 999. Extinguisher only if trained and it is safe. |
| First aid | Swallowed: do not make them sick, 999. Eyes: flush at least 10 minutes, then hospital. |

| Further action | Who | By when | Checked |
| --- | --- | --- | --- |
| Extinguisher, lock-up and DSEAR assessment done | DH | Before start |  |

| Assessed by Dan Hollins | Signed | Date 5 October 2026 | Next review 5 October 2027 |
| --- | --- | --- | --- |