COSHH Task Assessment Hollins & Reed Contracting Ltd

| Project or site House refurbishment, House 7 |  | Trade Drylining |  | Date assessed 5 October 2026 |  | Assessment ref CA-011 |
| --- | --- | --- | --- | --- | --- | --- |

| What the work makes |  |  |  |
| --- | --- | --- | --- |
| Task | Cutting, sanding joints | Material worked | Plasterboard, jointing compound |
| Dust or fume made | Gypsum dust, inhalable | Tool used | Handsaw, rasp, pole sander |
| Where | Indoors, one room at a time | How long a shift | About 3 hours |

| Exposure limit |  |  |  |
| --- | --- | --- | --- |
| WEL, 8-hour | Gypsum 10 mg/m³, respirable 4 | WEL, 15-minute | None listed |
| Monitoring result | Not measured | Measured on | Not measured |

| Who could be harmed | People doing it | Others nearby | Cleaners, fitters | Visitors, public | Higher risk |
| --- | --- | --- | --- | --- | --- |
| Dryliner, others, cleaner | ☒ | ☒ | ☒ | ☐ | ☐ |

Health effects

Breathing a lot of dust can damage the lungs and airways. Gypsum has very little silica, but board dust can carry some.

Jointing compound and plasters may hold quartz: read their sheet.

| Controls | In place | Still needed |
| --- | --- | --- |
| Another method | Tapered-edge boards. Handsaw and rasp, not a power saw. |  |
| On-tool extraction | Pole sander on an M class unit. |  |
| Water suppression | Not used on board. |  |
| Room ventilation | Window open, door shut to other rooms. |  |
| Fewer, for less time | Only the dryliner in the room while sanding. |  |
| Clean-up | Class M vacuum. No dry sweeping. |  |

| RPE: type and filter | Disposable | Half mask | Powered | Face-fit tested | Eye protection |
| --- | --- | --- | --- | --- | --- |
| APF 20 for sanding: disposable FFP3. | ☒ | ☐ | ☐ | ☒ | ☐ |

| Also needed | Yes | No | Detail | Next due |
| --- | --- | --- | --- | --- |
| Exposure monitoring | ☐ | ☒ | Extraction checked daily. Dust escaping: stop. |  |
| Health surveillance | ☐ | ☒ | Only if a cough or wheeze is reported. |  |
| LEV examination | ☒ | ☐ | Examined at least every 14 months. | March 2027 |
| Training | ☒ | ☐ | Extraction, clean shaven, clean-up. | Before start |

| Further action | Who | By when | Checked |
| --- | --- | --- | --- |
| Sander and M class unit on site | DH | Before start |  |

| Assessed by Dan Hollins |  | Signed |  | Date 5 October 2026 |  | Next review 5 October 2027 |
| --- | --- | --- | --- | --- | --- | --- |