COSHH Task Assessment Hollins & Reed Contracting Ltd

| Project or site House refurbishment, House 7 |  | Trade Groundworks |  | Date assessed 5 October 2026 |  | Assessment ref CA-007 |
| --- | --- | --- | --- | --- | --- | --- |

| What the work makes |  |  |  |
| --- | --- | --- | --- |
| Task | Cutting blocks, kerbs, flags | Material worked | Concrete block, kerb, flags |
| Dust or fume made | Respirable crystalline silica | Tool used | Cut-off saw with water feed |
| Where | Outdoors, marked cutting area | How long a shift | About 2 hours in short runs |

| Exposure limit |  |  |  |
| --- | --- | --- | --- |
| WEL, 8-hour | RCS 0.1 mg/m³, respirable | WEL, 15-minute | None listed |
| Monitoring result | Not measured | Measured on | Not measured |

| Who could be harmed | People doing it | Others nearby | Cleaners, fitters | Visitors, public | Higher risk |
| --- | --- | --- | --- | --- | --- |
| Operator, labourers nearby | ☒ | ☒ | ☐ | ☐ | ☐ |

Health effects

Silicosis, COPD and lung cancer: permanent disability and early death. The dust is too fine to see.

Dust on the skin and long contact with water may cause dermatitis.

| Controls | In place | Still needed |
| --- | --- | --- |
| Another method | Fewer cuts. Block splitter where it will do. | Order cut kerbs and flags next. |
| On-tool extraction | Not used on this saw. |  |
| Water suppression | Water on the diamond blade. Collect the slurry. |  |
| Room ventilation | Outdoors. Cut downwind of others. |  |
| Fewer, for less time | Only the operator in the cutting area. | Powered RPE if over 1 hour at a go. |
| Clean-up | Hose down, wet brush. No dry sweeping. |  |

| RPE: type and filter | Disposable | Half mask | Powered | Face-fit tested | Eye protection |
| --- | --- | --- | --- | --- | --- |
| APF 20: disposable FFP3. Waterproof trousers. | ☒ | ☐ | ☐ | ☒ | ☒ |

| Also needed | Yes | No | Detail | Next due |
| --- | --- | --- | --- | --- |
| Exposure monitoring | ☐ | ☒ | Only if the water fails or dust escapes. |  |
| Health surveillance | ☒ | ☐ | Questionnaire, lung function, chest x-ray, skin. | Baseline |
| LEV examination | ☐ | ☒ | None used. |  |
| Training | ☒ | ☐ | Water feed, clean shaven for RPE, clean-up. | Before start |

| Further action | Who | By when | Checked |
| --- | --- | --- | --- |
| Book baseline health checks before exposure | DH | Before start |  |

| Assessed by Dan Hollins |  | Signed |  | Date 5 October 2026 |  | Next review 5 October 2027 |
| --- | --- | --- | --- | --- | --- | --- |