COSHH Assessment Hollins & Reed Contracting Ltd

| Project or site House refurbishment, House 7 | Assessment ref CA-009 | Date assessed 5 October 2026 |
| --- | --- | --- |

| Substance |  |  |  |
| --- | --- | --- | --- |
| Product or substance | Solvent gloss, white spirit | Supplier | your supplier |
| SDS date | your sheet’s date | Form (liquid, dust…) | Liquid that gives off vapour |

| The work |  |  |  |
| --- | --- | --- | --- |
| Task | Gloss trim, clean brushes | Where | Bedrooms and landing |
| How often | Two days on this job | How long each time | Up to 4 hours a day |
| Amount used | About 5 litres, 2 litres spirit | Used with | Brushes, rollers, rags |

| Storage and waste |  |
| --- | --- |
| Storage | Sealed cans in the outdoor lock-up, away from heat and sun. Only the day’s cans indoors. |
| Handling | Lids back on straight away. Pour spirit outdoors over a tray. No smoking or flames near it. |
| Waste | Hazardous waste, not builders’ rubble: wet paint, used spirit and rags, sealed, by a registered carrier. Never down a drain. |

| Hazards (H codes) | Harmful or irritant | Corrosive | Toxic | Serious health hazard | Cancer or asthma | Flammable |
| --- | --- | --- | --- | --- | --- | --- |
| H226, H336, EUH066. Spirit adds H304. Some add H372. | ☒ | ☐ | ☐ | ☒ | ☐ | ☒ |

| Exposure limit |  |  |  |
| --- | --- | --- | --- |
| WEL, 8-hour | None in EH40 for white spirit | WEL, 15-minute | None listed |
| Monitoring result | Not measured | Measured on | Not measured |

| How it gets in | Breathing in | Skin | Eyes | Swallowing | Puncture |
| --- | --- | --- | --- | --- | --- |
| Vapour first, then skin | ☒ | ☒ | ☒ | ☒ | ☐ |

| Who could be harmed | People doing it | Others nearby | Cleaners, fitters | Visitors, public | Higher risk |
| --- | --- | --- | --- | --- | --- |
| Painters, other trades. Eczema or allergy. | ☒ | ☒ | ☐ | ☐ | ☒ |

Health effects

Flammable: vapour is heavier than air and can travel to a flame. Swallowed spirit can enter the lungs and kill.

Vapour causes headache and dizziness, a danger on steps. Your sheet may warn of harm from repeated breathing. Repeated contact dries and cracks skin. EH40’s example blend comes to 550 mg/m³: that is not a limit.

| Risk of ill health | Negligible | Remote | Possible | Probable | Minor | Significant | Serious |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Before controls | ☐ | ☐ | ☒ | ☐ | ☐ | ☐ | ☒ |
| With controls | ☐ | ☒ | ☐ | ☐ | ☐ | ☐ | ☒ |

COSHH Assessment Hollins & Reed Contracting Ltd

| Project or site House refurbishment, House 7 | Assessment ref CA-009 | Date assessed 5 October 2026 |
| --- | --- | --- |

| Controls | In place | Still needed |
| --- | --- | --- |
| Avoid or swap it | Water-based on walls and ceilings. Solvent on trim only. | Try water-based trim next job. |
| Change the method | Brush and roller, no spraying. Brushes cleaned outdoors. |  |
| Extract at source | Not used. Windows and the door open all the time. | Room that will not air: stop, ask. |
| Fewer, for less time | One painter to a room. Breaks in fresh air. |  |
| Washing, welfare | Soap and water at the cabin. Never spirit on hands. | Skin cream at the cabin. |

| PPE: type or standard | Gloves | Eye protection | RPE | Overalls | Boots |
| --- | --- | --- | --- | --- | --- |
| Nitrile, EN 374. Glasses to pour. | ☒ | ☒ | ☐ | ☒ | ☒ |

| If it goes wrong |  |
| --- | --- |
| Spill or leak | Flames out, windows open. Soak up with sand, bag it as hazardous waste. Keep it out of drains. |
| Fire | Raise the alarm, get out, ring 999. Only if safe, a foam, CO2 or powder extinguisher. No water jet. |
| Breathed in or swallowed | Fresh air and rest. Swallowed: do not make them sick. Rinse the mouth, go to hospital with the sheet. |
| On skin or in eyes | Skin: off with wet clothes, wash with soap and water. Eyes: rinse at least 15 minutes, get medical help. |

| Also needed | Yes | No | Detail | Next due |
| --- | --- | --- | --- | --- |
| Exposure monitoring | ☐ | ☒ | Two days, room aired. No WEL to test against. |  |
| Health surveillance | ☒ | ☐ | Skin checks by a responsible person. | Weekly |
| LEV examination | ☐ | ☒ | None used. |  |
| RPE face-fit test | ☐ | ☒ | None used. |  |
| Training, supervision | ☒ | ☐ | Fire, vapour, skin, rags, when to stop. | Before start |

| Further action | Who | By when | Checked |
| --- | --- | --- | --- |
| Foam or CO2 extinguisher to the landing | DH | Before start |  |
| Lidded metal rag bin, emptied outdoors daily | DH | Before start |  |
| Book a registered carrier for the waste | DH | Before start |  |

| Date | By | Review: reason and changes | Next review |
| --- | --- | --- | --- |
| 5 October 2026 | DH | First issue | 5 October 2027 |

| Assessed by Dan Hollins | Signed | Date 5 October 2026 | Shared with workers on 6 October 2026 |
| --- | --- | --- | --- |