COSHH Task Assessment Hollins & Reed Contracting Ltd

| Project or site House refurbishment, House 7 |  | Trade Joinery |  | Date assessed 5 October 2026 |  | Assessment ref CA-008 |
| --- | --- | --- | --- | --- | --- | --- |

| What the work makes |  |  |  |
| --- | --- | --- | --- |
| Task | Sawing, sanding joinery | Material worked | Softwood, oak, MDF |
| Dust or fume made | Mixed wood dust, inhalable | Tool used | Mitre saw, orbital sander |
| Where | Indoors, one room set aside | How long a shift | About 3 hours |

| Exposure limit |  |  |  |
| --- | --- | --- | --- |
| WEL, 8-hour | Mixed 3 mg/m³, softwood alone 5 | WEL, 15-minute | None listed |
| Monitoring result | Not measured | Measured on | Not measured |

| Who could be harmed | People doing it | Others nearby | Cleaners, fitters | Visitors, public | Higher risk |
| --- | --- | --- | --- | --- | --- |
| Joiner, others, cleaner | ☒ | ☒ | ☒ | ☐ | ☐ |

Health effects

Asthma and dermatitis. Hardwood dust can also cause a rare nasal cancer.

Keep exposure as low as reasonably practicable, even under the limit: these cause asthma and cancer.

| Controls | In place | Still needed |
| --- | --- | --- |
| Another method | Pre-cut, pre-finished timber where you can. | Order pre-finished skirting next. |
| On-tool extraction | H or M class unit on saw and sander. |  |
| Water suppression | Not used on wood. |  |
| Room ventilation | Windows open, door shut to other rooms. |  |
| Fewer, for less time | Only the joiner in the room while cutting. |  |
| Clean-up | Class M vacuum. No dry sweeping or airlines. |  |

| RPE: type and filter | Disposable | Half mask | Powered | Face-fit tested | Eye protection |
| --- | --- | --- | --- | --- | --- |
| APF 20: disposable FFP3. | ☒ | ☐ | ☐ | ☒ | ☐ |

| Also needed | Yes | No | Detail | Next due |
| --- | --- | --- | --- | --- |
| Exposure monitoring | ☐ | ☒ | Only if dust escapes the extraction. |  |
| Health surveillance | ☒ | ☐ | Asthma questionnaire, skin checks. | Baseline |
| LEV examination | ☒ | ☐ | Examined at least every 14 months. | Dec 2026 |
| Training | ☒ | ☐ | Extraction, clean shaven, clean-up. | Before start |

| Further action | Who | By when | Checked |
| --- | --- | --- | --- |
| Book baseline health checks before exposure | DH | Before start |  |
| Book the extractor’s examination | DH | 30 Oct 2026 |  |

| Assessed by Dan Hollins |  | Signed |  | Date 5 October 2026 |  | Next review 5 October 2027 |
| --- | --- | --- | --- | --- | --- | --- |