# Extension construction phase plan

# The job

| Your name and company | Hollins & Reed Contracting Ltd, Dan Hollins |
| --- | --- |
| Client name and address | Helen Tunstead, 9 Fernlea Rise, Stockport |
| Architect or principal designer | Marsden Lyle Architects Ltd, principal designer |
| What is the job? | Two-storey rear extension to a lived-in semi |
| Start, finish and other key dates | 8 June to 11 September 2026. Roof on by 24 July |
| Principal contractor, if more than one contractor | Hollins & Reed Contracting Ltd |

# What the client has told you   Services and isolation points, access limits, asbestos.

Stopcock under sink; gas meter, fuse box in porch. Side path access. Principal designer’s written appointment seen, 12 May 2026. Refurbishment asbestos survey, Hazel Bank Surveys Ltd, 19 May 2026: none in the kitchen, rear wall or eaves.

# Toilets, washing and rest   What there is, and where.

Unit on drive from day 1: toilet, basin, hot water, soap, towels, drinking water, heated room, table, seats, kettle, microwave.

# Who else is on site   Names, contact details, specialist companies, labourers.

Priya Shah; Joe Penketh, labourer. Roofer from 13 July: Cheadle Heath Roofing Ltd, Neil Hazlehurst, 0161 496 0167. Scaffolder: Hollins & Reed Scaffolding Ltd, 0161 496 0173. Electrician: Hollins & Reed Electrical Ltd, 0161 496 0174.

# Keeping everyone up to date   How changes reach everyone, such as a daily update.

Five-minute talk at 08:00 each day, with changes on the site board. Helen Tunstead hears the same day.

# Site rules   Protective equipment, parking, radios and phones, smoking, restricted areas, hot works.

Boots and hi-vis at all times, hard hat under the scaffold. Park on the road, never across a drive. No radios. No phones on the scaffold. No smoking on site or in the house. Ask Dan before going into a trench. No hot works.

# The main dangers on site

| Danger | Present? | The controls |
| --- | --- | --- |
| Falls from height | Yes | Guardrailed scaffold: inspected first, weekly, after gales |
| Collapse of excavations | Yes | Trenches supported and barriered, checked each shift |
| Collapse of structures | Yes | Props to the engineer’s drawing, set by a competent person |
| Exposure to building dusts | Yes | Wet cutting, extraction on tools, masks, no dry sweeping |
| Exposure to asbestos | No | None in the survey. Stop and ring Dan if anything looks odd |
| Activities or workers requiring supervision | Yes | Joe Penketh is new to us and stays with Priya Shah |
| Electricity | Yes | Supply off before drilling walls; buried services traced |
| Risks to members of the public, the client and others | Yes | Site fenced off from the garden; scaffold netted; chute to skip |
| Other dangers on site | Yes | Deliveries watched on the drive by a second person |

# Running the job safely

| Person responsible for the job running safely | Dan Hollins, 0161 496 0142 |
| --- | --- |
| How supervision will be provided | Dan Hollins daily, Priya Shah if he is away |
| Plan drawn up by, and date | Dan Hollins, 1 June 2026 |