Confined Space Entry Permit Hollins & Reed Contracting Ltd

| Permit no. HR-0420 | Site Unit 3, Canal Side Business Park | Exact location Foul manhole MH2, east yard |
| --- | --- | --- |

| Details |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Contractor | Hollins & Reed | Issued to | Priya Shah | Valid from | 15/10/26 09:00 |
| Valid to | 15/10/26 14:00 | RAMS ref | HR-RAMS-10 | Isolation cert | Stopper T1 |

| The work |  |
| --- | --- |
| Space and entry point | Foul manhole MH2, 2.4 m deep, 1.2 m across. One entry, the top cover. |
| Work to be done | Join the new drain through the chamber wall. Two to lift the pipe. Make good the benching. |
| Why entry, time limit | The joint cannot be made from the surface. Two entries, 45 minutes each. |

| Hazards |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Oxygen low or high | ☒ | Toxic gas | ☒ | Flammable air | ☒ | Engulfment | ☒ |
| Heat | ☐ | Drowning | ☒ | Restricted entry | ☒ | Power, machinery | ☐ |

| Precautions | Done | By |
| --- | --- | --- |
| Space isolated and locked off | ☒ | Priya Shah, tied off |
| Cleaned, purged and ventilated | ☒ | Reece Ollerton, fan on |
| Air tested before entry and during | ☒ | Priya Shah |
| Supervisor and top person named | ☒ | Dan Hollins, Priya Shah |
| Communication agreed | ☒ | Radio and line signals |
| Rescue plan and equipment ready | ☒ | Dan Hollins, drilled |
| Harness and line | ☒ | Both entrants |
| Safe lighting and tools | ☒ | Rated for the space |
| Breathing apparatus where needed | ☐ | Not needed. Escape sets |
| How emergency services are called | ☒ | Priya Shah, 999 |

| What is isolated | How | Locked | Tag | Proved by |
| --- | --- | --- | --- | --- |
| Incoming foul drain | Stopper at MH1, tied off | ☐ | T1 | Priya Shah |

| Equipment |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Gas monitor | ☒ | Tripod and winch | ☒ | Escape BA | ☒ | Radio | ☒ |
| Lighting | ☒ |  | ☐ |  | ☐ |  | ☐ |

Other conditions

Out on any alarm, falling oxygen or a reading moving towards a limit. Our limits CO 20 ppm, H2S 5 ppm, gas 10% LEL.

Test oxygen, then gas, then toxics, from the top by probe. Top person stays out. Rescue by winch from the surface.

| Sign-off | Name | Signature | Date | Time |
| --- | --- | --- | --- | --- |
| Issued: precautions checked | Dan Hollins | signed | 15/10/26 | 08:55 |
| Accepted: permit understood | Priya Shah | signed | 15/10/26 | 09:00 |

Confined Space Entry Permit Hollins & Reed Contracting Ltd

| Permit no. HR-0420 | Site Unit 3, Canal Side Business Park | Exact location Foul manhole MH2, east yard |
| --- | --- | --- |

| Gas tests |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Time | O2 % | LEL % | CO ppm | H2S ppm | By |
| 08:40 | 20.8 | 0 | 0 | 1 | Priya Shah |
| 09:10 | 20.9 | 0 | 0 | 0 | Priya Shah |
| 09:50 | 20.9 | 0 | 0 | 0 | Priya Shah |
| 11:00 | 20.8 | 0 | 0 | 1 | Priya Shah |
| 11:40 | 20.9 | 0 | 0 | 0 | Priya Shah |

| People in the space: name | Company | Briefed | In | Out |
| --- | --- | --- | --- | --- |
| Reece Ollerton | Hollins & Reed | ☒ | 09:15 | 09:55 |
| Joe Penketh | Hollins & Reed | ☒ | 09:15 | 09:55 |
| Reece Ollerton | Hollins & Reed | ☒ | 11:05 | 11:45 |
| Joe Penketh | Hollins & Reed | ☒ | 11:05 | 11:45 |
|  |  | ☐ |  |  |
|  |  | ☐ |  |  |

| Cancellation |  |
| --- | --- |
| All out at | 11:45. Both entrants counted out. |
| Space secured | Cover back on at 12:10. Stopper out at MH1, drain running. |
| Equipment removed | Tripod, fan, monitors and lines off the yard by 12:30. |

| Handback |  |
| --- | --- |
| Isolations removed | Stopper T1 removed by Priya Shah at 12:05 |
| Area returned to | Thornley Vale Estates Ltd, site keyholder |
| Notes | Readings in the job file. Both entrants washed down after each entry. |

| Sign-off | Name | Signature | Date | Time |
| --- | --- | --- | --- | --- |
| Extended: new end time | Not extended |  |  |  |
| Cancelled: area checked | Dan Hollins | signed | 15/10/26 | 12:35 |
| Handed back: area returned | Dan Hollins | signed | 15/10/26 | 12:40 |