Permit to Work Hollins & Reed Contracting Ltd

| Permit no. HR-0421 | Site High street bank branch | Exact location Banking hall, reception end |
| --- | --- | --- |

| Details |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Contractor | Hollins & Reed | Issued to | Priya Shah | Valid from | 19/10/26 18:30 |
| Valid to | 20/10/26 05:30 | RAMS ref | HR-RAMS-11 |  |  |

| The work |  |
| --- | --- |
| Work to be done | Take out the old reception desk and two partitions. Fit the new ones from drawing HR-D-05. |
| Plant and equipment | Hand tools, cordless drivers, sack truck. Panels arrive cut to size: no cutting in the hall. |

| Hazards |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Services | ☒ | Height | ☐ | Confined space | ☐ | Hot work | ☐ |
| Lifting | ☐ | Traffic | ☐ | COSHH substances | ☐ | Other | ☒ |

| Precautions | Done | By |
| --- | --- | --- |
| Intruder alarm zone 2 isolated, fire alarm stays on | ☒ | Alarm engineer |
| Power to the old desk isolated and proved dead | ☒ | Bank's electrician |
| Exits and the rear corridor kept clear all night | ☒ | Reece Ollerton |
| Security told who is in, and their number on this permit | ☒ | Dan Hollins |
| No hot work: the bank allows none on this permit | ☒ | Dan Hollins |
| Waste out by the rear door only, door shut after each trip | ☒ | Joe Penketh |

| What is isolated | How | Locked | Tag | Proved by |
| --- | --- | --- | --- | --- |
| Intruder alarm zone 2 | Bypassed at the panel | ☐ | A1 | Alarm engineer |
| Desk ring circuit | Way 9, breaker locked off | ☒ | E1 | Bank's electrician |

| People on this permit: name | Company | Briefed |
| --- | --- | --- |
| Priya Shah | Hollins & Reed | ☒ |
| Reece Ollerton | Hollins & Reed | ☒ |
| Joe Penketh | Hollins & Reed | ☒ |

Other conditions

One night, 18:30 to 05:30. Not finished by 05:00: stop, make safe, ring the duty manager. Intruder zone 2 bypassed 18:15, desk circuit proved dead 18:20, both before issue.

Other hazard: out of hours, one security officer alone. Security 0161 496 0178. Check in at 18:30, at 00:00 and on leaving. Nobody past the counter line.

Asbestos: none in the desk or partitions on the bank's register. Anything unexpected in the walls, stop and ask.

| Sign-off | Name | Signature | Date | Time |
| --- | --- | --- | --- | --- |
| Issued: precautions checked | Dan Hollins | signed | 19/10/26 | 18:25 |
| Accepted: permit understood | Priya Shah | signed | 19/10/26 | 18:28 |

Permit to Work Hollins & Reed Contracting Ltd

| Permit no. HR-0421 | Site High street bank branch | Exact location Banking hall, reception end |
| --- | --- | --- |

| Cancellation |  |
| --- | --- |
| Work stopped at | 04:50. New desk and both partitions fitted. |
| Area checked | Hall swept and walked at 05:00. Exits clear. Rear door shut, checked by security. |
| Equipment removed | Tools and waste out of the rear door by 04:58. |

| Handback |  |
| --- | --- |
| Isolations removed | Desk circuit back on at 05:05, by the bank's electrician. Intruder alarm zone 2 reinstated and tested at 05:10, by the alarm engineer. |
| Area returned to | The bank's duty manager, 05:10 |
| Notes | Checked in with security 18:30, 00:00, 05:06. The bank's electrician connected and tested the new desk sockets before switching back on. |

| Sign-off | Name | Signature | Date | Time |
| --- | --- | --- | --- | --- |
| Extended: new end time | Not extended |  |  |  |
| Cancelled: area checked | Dan Hollins | signed | 20/10/26 | 05:08 |
| Handed back: area returned | Dan Hollins | signed | 20/10/26 | 05:10 |