Permit to Dig Hollins & Reed Contracting Ltd

| Permit no. HR-0419 | Site Unit 3, Canal Side Business Park | Exact location East yard, gully to manhole MH2 |
| --- | --- | --- |

| Details |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Contractor | Hollins & Reed | Issued to | Priya Shah | Valid from | 14/10/26 08:00 |
| Valid to | 14/10/26 16:30 | RAMS ref | HR-RAMS-09 | Drawing, date | HR-D-02, 05/10/26 |

| The work |  |
| --- | --- |
| Where, depth, method | Trench 6 m long, 1.2 m deep. Breaker through concrete, then mini excavator with a spotter. |
| Hand dig zone | Hand tools only near marked lines. Breaker 500 mm off them. Insulated tools by the cable. |

| Hazards |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Electricity | ☒ | Gas | ☒ | Water | ☒ | Telecoms | ☐ |
| Drains and sewers | ☒ | District heating | ☐ | Contaminated ground | ☐ | Unstable ground | ☒ |
| Water ingress | ☒ | Traffic | ☒ | Services unknown | ☐ |  | ☐ |

| Precautions | Done | By |
| --- | --- | --- |
| Drawings obtained and dated | ☒ | Dan Hollins, 05/10/26 |
| Scanned with locator and signal generator | ☒ | Dan Hollins, 13/10/26 |
| Services marked on the plan and ground | ☒ | Dan Hollins |
| Trial holes hand dug | ☒ | Priya Shah, two holes |
| Services exposed and supported | ☐ | Not needed. None crossed |
| Edge protection and ladders in place | ☒ | Reece Ollerton |
| Spoil kept back from the edge | ☒ | Reece Ollerton |
| Sides supported or battered | ☒ | Priya Shah, supports |
| Inspected at the start of each shift | ☒ | Dan Hollins |
| Gas monitoring, traffic management | ☒ | Reece Ollerton, barriers |

| Services made dead | How | Locked | Tag | Proved by |
| --- | --- | --- | --- | --- |
| Nothing isolated | All services stay live | ☐ |  |  |

| People on this permit: name | Company | Briefed |
| --- | --- | --- |
| Priya Shah | Hollins & Reed | ☒ |
| Reece Ollerton | Hollins & Reed | ☒ |

Other conditions

Treat every service as live. Re-scan as the trench deepens. Unknown service, stop and ring 0161 496 0142.

Cable struck, driver stays in the cab. Nobody goes near until the owner makes it safe.

| Sign-off | Name | Signature | Date | Time |
| --- | --- | --- | --- | --- |
| Issued: precautions checked | Dan Hollins | signed | 14/10/26 | 07:45 |
| Accepted: permit understood | Priya Shah | signed | 14/10/26 | 07:50 |

Permit to Dig Hollins & Reed Contracting Ltd

| Permit no. HR-0419 | Site Unit 3, Canal Side Business Park | Exact location East yard, gully to manhole MH2 |
| --- | --- | --- |

| Excavation inspections |  |  |  |
| --- | --- | --- | --- |
| Date | Time | By | Result |
| 14/10/26 | 13:00 | Dan Hollins | Trench at 0.9 m. Supports in, sides sound, barriers up. |
| 15/10/26 | 07:45 | Dan Hollins | Start of shift, after rain. Pumped out. Sides sound. |
| 15/10/26 | 12:45 | Dan Hollins | Before backfilling. Sides, supports and barriers sound. |

| Cancellation |  |
| --- | --- |
| Backfilled or secured | Backfilled in layers. Concrete reinstated 15 October. No concrete around services. |
| Services as found | Cable and gas pipe where the drawing shows. Neither touched. |
| Drawings marked up | New drain added to HR-D-02. Copy to the site plan. |

| Handback |  |
| --- | --- |
| Isolations removed | None were made |
| Area returned to | Thornley Vale Estates Ltd, site keyholder |
| Notes | Yard open again. Barriers gone. |

| Sign-off | Name | Signature | Date | Time |
| --- | --- | --- | --- | --- |
| Extended: new end time | Dan Hollins, to 15/10 16:00 | signed | 14/10/26 | 16:15 |
| Cancelled: area checked | Dan Hollins | signed | 15/10/26 | 15:30 |
| Handed back: area returned | Dan Hollins | signed | 15/10/26 | 15:40 |