Working at Height Permit Hollins & Reed Contracting Ltd

| Permit no. HR-0422 | Site Unit 5, Canal Side Business Park | Exact location Flat roof, south section, inside the barrier |
| --- | --- | --- |

| Details |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Contractor | Hollins & Reed | Issued to | Owen Tattersall | Valid from | 22/10/26 07:45 |
| Valid to | 22/10/26 12:30 | RAMS ref | HR-RAMS-12 |  |  |

| The work |  |
| --- | --- |
| Where and how reached | Large felt roof, concrete deck. Fixed ladder to a hatch inside the barrier, 9 m from the edge. |
| Work to be done | Set the barrier. Two-hour condition survey, photos and moisture readings. Strike the barrier. |
| Why not from ground | Pole camera pictures on 15 October could not show if the felt is wet. That needs a meter. |

| Hazards |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fragile roof | ☒ | Unprotected edge | ☒ | Roof lights | ☒ | Weather | ☒ |
| Overhead lines | ☐ | People below | ☒ | Lone working | ☐ | Falling objects | ☒ |

| Precautions | Done | By |
| --- | --- | --- |
| Access equipment inspected | ☒ | Dan Hollins |
| Edge protection in place | ☐ | Barrier 2 m instead |
| Fragile surfaces covered or barriered | ☒ | Barrier, by 08:50 |
| Harness and anchor points | ☐ | Not needed. No anchor |
| Rescue plan in place | ☒ | See below |
| Exclusion zone below | ☒ | Keyholder, cones |
| Weather checked | ☒ | Readings, sheet 2 |
| Tools tethered | ☒ | Wrist straps |
| Lighting | ☐ | Not needed. Daylight |

| Equipment |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MEWP | ☐ | Tower | ☐ | Scaffold | ☐ | Harness | ☐ |
| Netting | ☐ |  | ☐ |  | ☐ |  | ☐ |

| People on this permit: name | Company | Briefed |
| --- | --- | --- |
| Owen Tattersall, Joe Penketh: survey | Hollins & Reed | ☒ |
| Reece Ollerton, Priya Shah: barrier | Hollins & Reed | ☒ |

Other conditions

Issuer at the hatch while anyone works. Stay inside the barrier. Stop at a steady 8 m/s, our limit for this job, or on rain.

Rescue: no harness. Hurt or ill: ring 999, say fixed ladder, may need fire service. Issuer meets them at the hatch.

| Sign-off | Name | Signature | Date | Time |
| --- | --- | --- | --- | --- |
| Issued: precautions checked | Dan Hollins | signed | 22/10/26 | 07:35 |
| Accepted: permit understood | Owen Tattersall | signed | 22/10/26 | 07:40 |

Working at Height Permit Hollins & Reed Contracting Ltd

| Permit no. HR-0422 | Site Unit 5, Canal Side Business Park | Exact location Flat roof, south section, inside the barrier |
| --- | --- | --- |

| Weather and equipment checks |  |  |  |
| --- | --- | --- | --- |
| Time | Wind | By | Result |
| 07:30 | 2.4 m/s | Reece Ollerton | Read at the hatch, before issue. Dry. Under our limit. |
| 07:45 | 2.6 m/s | Reece Ollerton | All four briefed by the issuer. Barrier team up. Our method for this job: sections and weights hauled through the hatch on a rope, nobody on the ladder while a load moves. Line taped out from the hatch first, sections set from inside it, one setting, one watching. Nobody past the tape. |
| 08:50 | 2.9 m/s | Dan Hollins | Barrier done, weighted, 2 m from edges and roof lights. Team stopped while the issuer walked it. Team down. |
| 09:00 | 3.1 m/s | Joe Penketh | Survey team up. Dry. Under our limit. |
| 10:00 | 4.4 m/s | Joe Penketh | Dry. Both inside the barrier. Issuer at the hatch. |
| 11:00 | 6.3 m/s | Joe Penketh | Wind rising, cloud building. Last readings, then off the roof. |
| 11:20 | 6.6 m/s | Reece Ollerton | Under our limit. Barrier team up. Sections lifted back towards the hatch, nobody past the line. Issuer at the hatch. |
| 11:55 | 7.1 m/s | Reece Ollerton | Last section lowered through the hatch. Team down. |

| Cancellation |  |
| --- | --- |
| Work stopped at | 11:55. Survey done 11:10, barrier struck 11:55. All four down and counted. |
| Area left safe | Hatch shut and locked 12:00. Nothing left on the roof. |
| Equipment removed | Barrier sections to the van. Cones lifted 12:05. |

| Handback |  |
| --- | --- |
| Area returned to | Thornley Vale Estates Ltd, site keyholder, 12:15 |
| Notes | Survey report to follow. Not part of this permit. |

| Sign-off | Name | Signature | Date | Time |
| --- | --- | --- | --- | --- |
| Extended: new end time | Not extended |  |  |  |
| Cancelled: area checked | Dan Hollins | signed | 22/10/26 | 12:10 |
| Handed back: area returned | Dan Hollins | signed | 22/10/26 | 12:15 |